TWO POSSIBLE CONFIGURATIONS FOR GSDR VAC ANS CW SKIMMER

- 2 VAC cables only.

- PC sound card (IDT Hi def+MME) + One VAC cable

GSDR CONFIGS COMMON TO BOTH SOLUTIONS:

VAC PANEL CONFIGURATION

TWO VAC CABLE CONFIG PROPOSED BY FLEX RADIO

WDM-KS works with lower latency than MME (change latency if popping)

Problem w. above: whole GSDR frequency display moves with mouse freq. choices when RX shift enabled (LO changes with frequency).

Frequencies must be selected only by clicking on Skimmer waterfall display or decoded calls.

CW Skimmer setups with 2 VAC cables

Remarx: GSDR CAT com9 connected to DDUtil com19 via virtual data cable

Attention: if after a click to a station on Skimmer waterfall the station signals/call is not seen/heard within the green cursor, reverse the I/Q value in the Audio setting of the Skimmer!

Skimmer is connected to DDUtil RCP2 via com 13<->com14 virtual data cable

CW SKIMMER ACTIVATED

Curved pattern comes from evaluation VAC version (voice reminding message); disappeared after installation of registered version of VAC

After installing registered version of VAC (no more voice message interference)

GSDR WINDOW SHOWING SAME FREQUENCY ON WHICH SKIMMER WAS TUNED TO

SYSTEM USING PC SOUNDCARD AND 1 VAC CABLE

PROPOSED BY Goran YT7PWR

Works fine, Skimmer indicates correct frequencies, but on GSDR screen still moving frequency display due to variation of LO with frequency selection.. Frequencies must be selected only by clicking on Skimmer waterfall display or on decoded calls.

Above: RX shift enabled causes moving frequency display on GSDR. Use Skimmer to select frequencies/stns.

SKIMMER SETUP

Attention: if after a click to a station on Skimmer waterfall the station signals/call is not seen/heard within the green cursor, reverse the I/Q value in the Audio setting of the Skimmer!

Remarx: GSDR CAT com9 connected to DDUtil com19 via virtual data cable

Skimmer connected to DDUtil RCP2 via com 13<->com14 virtual data cable

CW Skimmer full span = selected frequency +/- 12 kHz

(sensitivity drops at edges due to Skimmer filter).

Problem: If I want to set QSX frequency for TX split, I lose listening of DX-ped's frequency. This becomes even more complicated when the QSX is far above the 12 kHz  off-VFO limit of the Skimmer.

Possible solution: select SPLIT on GSDR , click MS (memory save) to save DX-ped freq, jump to Skimmer, with PC PgUP and mouse click on waterfall, select TX Split frequency, which will appear in GSDR as vfoA frequency, then select A>B to transfer this frequency to TX (vfo B), then click on MR (memory return) to continue listening DX station and transmitt on a new Split frequency. (Must use VFO MEM instead of freq swap A<>B as with some large RX/TX differences swap risks to result in both frequencies identical, even with “VFO B extended” is checked in GSDR).

To select new QSX frequency, click on Skimmer Waterfall and again on MR to receive DX freq. In this solution listening to DX-ped frequency is stil performed via G11/GSDR…

DDUTIL CONFIGURATION

CONCLUSION

After all trials and head kicking, I think the only viable solution would be to drop listening on SDR, leave SDR only for Skimmer, and do RX and TX on  FT1000MP, with SPLIT button activated. This will allow for QSX beyond the +12 kHz limit of the Skimmer:

Simple and fast solution:

With SPLIT activated on FT1000MP, listen to DX-ped frequency on sub-RX vfo-B, transmit on vfo-A. For better CW selectivity install optional 500Hz CW filter YF-115C for sub-RX.

To listen to DX-ped on better performing main RX (vfo-A):

Spot DX-ped on Skimmer and click , it will select vfo-A on MP. Then press STO (under QMB) and select QSX frequency on Skimmer, which will be synced to vfo-A. Then press A>B and RCL (under QMB), to continue listening to DX on vfo-A and transmit on QSX vfo-B.

Next QSX goes to vfo-A, A>B + RCL , and so on...

Ideal solution: modify DDUtil program to allow selection of vfo-A or vfo-B for Slave Radio. This would allow for listening on main RX and automatic selection of vfo-B via Skimmer!